# The Transition Matrix Formalism for the Scattering of a Buried Pipeline in an Elastic-Half Space

Wen-I Liao1, Jenn-Shin Hwang 2, Shiang-Jung Wang 3

1 Department of Civil Engineering, National Taipei University of Technology, Taipei, Taiwan 
2 Department of Construction Engineering, National Taiwan University of Science and Technology, Taipei, Taiwan
3 National Center for Research on Earthquake Engineering, Taipei, Taiwan

Abstract: It is investigated in this paper on the transition matrix formulation for the analysis of responses of an elastic half-space with a buried pipeline subjected to obliquely incident waves. The basis functions are constructed using the moving P-, SV-, and SH-wave source potentials to represent the scattered and refracted wave fields in a series form. The associated T-matrix expression of the elastic inclusion is derived using Betti’s third identity. Typical numerical results by obliquely incident plane waves are obtained for verification purpose.

Keywords: Transition matrix, Betti’s identity, Scattering, plane wave

## 1 Introduction

The transition matrix (T-matrix) theory for the scattering of acoustic waves by a single scatterer in an infinite medium was first proposed by Waterman [1, 2]. Initially, the theory was derived using Helmholtz integral formula. Pao [3, 4] has shown that the derivation can be greatly simplified if Green’s identity is applied to the acoustic wave problem as well as Betti’s third identity is applied to the elastic wave problem. Since then, there have been numerous publications on the applications and extensions to electromagnetic and elastic wave propagation problems. Detailed references for electromagnetic and elastic wave propagation problems can be found in Varatharajula and Pao [5] and Varadan and Varadan, [6]). Yeh and Pao [7] have recently proposed the T-matrix formulation for the scattering of acoustic waves by a multiple layered inclusion in an infinite medium. Chai et al. [8] have successfully employed the cylindrical wave functions as basis functions for SH-wave scattering in a two-dimensional alluvial valley to study its resonant phenomenon according to the property of the T-matrix. However, it should be noted that only Neuman’s boundary condition can be satisfied by the angular part of the cylindrical wave function.

As to research on dynamic responses of a buried layered inclusion (tunnel or pipeline) in an elastic semi-infinity medium subjected to incident plane waves, Lee and Trifunac [9] have studied the scattering of two-dimensional anti-plane problem using the method of series expansion. The cylindrical pipeline subjected to incident SH-waves was also analyzed and discussed. El-Akily and Datta [10] have used the Flugge shell theory to simplify the behavior of the cylindrical pipeline, and expand the scattering fields of the exterior region by circular cylindrical wave functions. Wong et al. [11] combined the finite element method with the wave function expansion to analyze the responses of a buried pipeline with non-circular cross-sections. Luco and de Barros [12] adopted the indirect boundary integral method to construct the scattering field and combined it with the Donell shell theory to study the responses of a cylindrical pipeline subjected to obliquely incident plane waves. Yeh et al. [13] have recently proposed a hybrid method for analyzing the dynamic responses of shells buried in an elastic half-plane. More recently, Chen et al. [14, 15] used the boundary integral method with degenerating kernel of fundamental solutions to transform the improper boundary integrals to a series of sums for the study of the exterior radiation and scattering problems with circular boundaries.

In this paper, Yeh and Pao’s [7] concept is extended to analyze the scattering of elastic waves by a lined pipeline subjected to obliquely incident waves in an elastic half-space. The basis functions and their regular parts are constructed using moving P-, SV- and SH-wave source potentials, Betti’s third identity and orthogonality conditions for the elastic half-space. The T-matrix relating the coefficient of the scattered waves to those of the free field is developed among the basis functions. Although the theory presented herein is only for a buried pipeline in an elastic half-space, the extension theory for the multiple layered inclusion is straightforward and easy to derive. Finally, numerical results for the stress and displacement components of a buried pipeline within the elastic half-space with different obliquely incident waves are calculated.

Figure 1  Configuration and coordinate system. (a) section view (b) plan view.

## 2 Statement of Problems and Basis Functions

Considering a layered elastic inclusion (pipeline) embedded in an elastic half-space D given in Fig. 1, the interface between the elastic half-space and the inclusion surface is denoted as S1. The elastic inclusion is divided into two parts denoted as D1 and D2 with an interface S2. For the half-space D, the density and Lame' constants are,  and , respectively, and those for the domain Dj (j=1,2) are denoted correspondingly as ,  and . The elastic half-space is subjected to obliquely incident plane waves with harmonics to the time variable t and to the space variable y. The vertical and horizontal incident angles of the incident waves are  and , respectively. A factor  is assumed throughout this paper for the waves with specified circular frequency  and wave-number ky which is the y-component wave-number of the incident waves. The wave-number ky for different types of incident waves are listed in the follows

Incident P-wave:

Incident S-wave:

Incident Rayleigh wave:

where  and  corresponding to the longitudinal and shear wave velocities  and ,  and  is with respect to the Rayleigh wave velocity .

As the incident wave impinges on the elastic inclusion, one part is transmitted into domains  and , and the other is reflected back into the half-space. Hence the wave field in the half-space comprises the free field  and the scattered field . Then the total displacement field for the half-space can be expressed as

,   	(1)

The scattered field can be represented by suitable independent sets of basis functions satisfying the free-surface and radiation conditions at infinity. The basis functions adopted in this study are singular solutions of the responses of a half-space subjected to moving P-, SV- and SH-wave source potentials and their high-order terms, and are expressed in integral forms. The detailed derivation of those basis functions is available in [16]. The scattered wave field of the exterior region is expressed as

(2)

where  (=p,,h) is the basis functions for the elastic half-space, the superscript  and h denotes the wave fields generated by moving P-, SV- and SH-wave source potentials, respectively. The subscript m ranges from 0 to N in which N is the approximated order. Similarly, the displacement field of the free field can be expanded into a series of regular basis function as

(3)

where  is the regular part of the basis function . For domains  and , the refracted wave fields can be expressed as

,                       (4)

,                                    (5)

where (j = 1, 2; α = p, v, h) is the regular part of the basis function  of domain Dj. The basis function  for the half-space should satisfy the traction-free conditions on the free surface, but the basis function  for the elastic inclusion needs not satisfy the traction-free conditions. Hence, the basis functions for domains  and  include only the source term of the source potentials and have the following component form

;　, ,                 (6)

where . For (P-wave source potential), components of  are given by

(7)

For (SV-wave source potential), components of  are

(8)

For (SH-wave source potential), components of  are

(9)

where , and  is properly selected as

(10)

where  and  are Chebyshev polynomials of the first and second kinds, respectively. The basis functions (j = 1, 2; = p, v, h) can be easily evaluated using the method presented in Section 4. By applying Betti's third identity of elasticity

(11)

In which  is the surface of volume ,  and  are tractions of displacement fields  and , respectively. The orthogonality conditions for domain D are

(12)

(13)

(14)

where  can be any arbitrary closed bounded surface that outside the domain surface . Similarly, the orthogonality conditions for domain  are

(15)

(16)

(17)

where  can be any arbitrary closed surface inside domain  as described by Figure 2.

Figure 2  Domains and relative surfaces of this problem.

## 3. Transition Matrix Formulism

If the elastic half-space is assumed to be continuous with the elastic inclusion, the continuity conditions on the interface  are

(18)

where  and  are the total displacements on the interface S, approaching respectively from the positive and negative directions of the unit normal. () and  () are the associated tractions of  and , respectively.  is the component of unit outer normal vector  of surface . For interface  between domains  and , in the case that domain  is an elastic medium and continuous with , the boundary conditions on surface  are

（19）

In the case that domain  is a cavity, then the boundary conditions are

(20)

In this study, domain  is presumed to be a cavity. This is simply because to represent a buried lined pipeline or a pipeline by assuming domain  as a cavity is more meaningful and has more applications than as an elastic medium.

If the volume in Eq. (11) is the domain bounded by surfaces  and  shown in Fig. 2, and let , then

(21)

Substituting Eqs. (2) and (3) into the right-hand side of Eq. (21), and applying the orthogonality condition of Eq. (12), Eq.(21) can then be reduced to the following form

(22)

From the continuity conditions of Eq.(18), the coefficient vector  can be expressed as

(23)

where

(24)

(25)

For the same domain bounded by surfaces  and , letting  yields

(26)

Substituting the expansion form of Eq. (3) of free-field displacement into the right-hand side of Eq. (26) and using the orthogonality condition, Eq.(26) can be reduced to the following form

(27)

where the matrix  is defined as

(28)

In addition, from the continuity conditions of Eq.(18), the matrix  is reduced to

(29)

In which

(30)

(31)

Now, For volume V in Eq. (11) enclosed by surfaces  and  of Fig. 2, letting deduces

(32)

Applying the orthogonality condition of Eq.(15), Eq.(32) can be reduced to

(33)

where matrix  is defined as

(34)

and Eq.(33) can be rewritten as

(35)

If domain  is a cavity, from the boundary conditions of Eq. (20), we can obtain the relation between coefficients  and  as

(36)

where

(37)

Now, for the domain enclosed by surfaces  and , letting  yields

(38)

From the boundary conditions of Eq.(20), Eq.(38) becomes

(39)

where

(40)

In summary, the relations between the unknown coefficients  and  in Eqs. (23), (29), (36) and (39) can be expressed in matrix forms as the follows

(41)

(42)

(43)

(44)

Then the relation between the coefficient vector  and vector  is given by

(45)

where

(46)

This is the T-matrix relating the coefficient of the scattered waves to that of the free field for the elastic half-space with layered inclusions.

## 4 Numerical Examples

The problems of a cylindrical pipeline with  buried at different depths and subjected to incident plane waves are investigated in the follows.  and  are the outer and inner radius of the pipeline, respectively. For all cases included in this study, the Poisson ratio for the pipeline and for the elastic half-space were assumed to be 0.2 and 1/3, respectively. The ratio of density of the pipeline to the half-space was taken as . The ratio of shear modulus was set to . The excitation frequency of incident wave was normalized as  to be consistent with other studies. Basis functions of the half-space are calculated by the method of steepest descend [16]. The choice of approximate order N of wave series are based on the convergence of calculated results. In all numerical examples of this study, N=5 is enough to have a satisfactory results as compared with those results with more high order terms. For the calculations of surface integrals in Equations (24), (31), (34), (37), and (40) are accomplished by Simpson’s rule with integration points M =61.

To verify the validity and accuracy of the T-matrix formulation in this study, the plane strain cases were first studied and the results obtained were compared with those obtained by Yeh et al. [17]. Figures 3 and 4 show the calculated displacement amplitudes and hoop stresses along the inner surface of the cylindrical pipeline buried at different depths respectively subjected to incident P-wave and SV-wave (). The results determined by the least square method (Yeh et al., [17]) are denoted by circles in these figures, and it can be seen that the agreement is excellent. For the responses of a buried pipeline subjected to obliquely incident waves, Figs. 5 and 6 show the displacements and stresses along the inner surface of the cylindrical pipeline subjected to obliquely incident P-wave and SH-wave (, , ) with buried depths of  and . In these figures, the displacement () is normalized with respect to the amplitude  of incident wave, and the stress fields are normalized with respect to the factor . The responses in Figs. 5 and 6 should be symmetric with respect to the vertical z-axis ( and ) because the wave is propagating in the direction of the axis of the pipeline. The effect of the embedment depth of the pipeline on the response can also be found in Figs. 5 and 6. Obviously, the embedment depth has a significant effect on the response and, particularly, on the case of incident P-wave.

## 5 Conclusions

The T-matrix, which relates the unknown scattering coefficients to the free-field coefficients, can be derived directly by the orthogonality conditions as well as the continuity conditions in the interface between the buried two-layered scatterer and the surrounding half-space. The basis functions and their regular parts are constructed in a systematic manner for the problems of obliquely incident waves. The results for wave scattering of a buried pipeline subjected to incident plane waves were calculated and verified. Although only a limited number of numerical results are presented in this paper, applications of the proposed method may be extended to problems of wave scattering in multiple layered scatter, embedded cavities, surface scatter and other problems of oblique incidence. In addition to elastic waves, the proposed T-matrix method could also be applicable to coupled field problems, such as thermoelasticity and poroelasticity.

## Acknowledgement

The authors gratefully acknowledge the financial support granted by the National Science Council, R.O.C., the facilities provided by the National Taipei University of Technology is also highly appreciated.

## References

1 Waterman, P. C., 1969, New Formulation of Acoustic Scattering,'' Journal of the Acoustical Society of America, Vol. 45, No. 6, pp. 1417-1429.

2 Waterman, P. C., 1976, Matrix Theory of Elastic Wave Scattering,'' Journal of the Acoustical Society of America, Vol. 60, No. 3, pp. 567-580.

3 Pao, Y. H., 1978, Betti's Identity and Transition Matrix for Elastic Waves, Journal of the Acoustical Society of America, Vol. 64, No. 1, pp. 302-310.

4 Pao, Y. H., 1978, The Transition Matrix for the Scattering of Acoustic Waves and Elastic Waves,'' Proceeding of the IUTAM Symposium on Modern Problems in Elastic Wave Propagation, (ed. J. Miklowitz and J. Achenbach), Wiley, New York, 123-144.

5 Varatharajulu, V. and Pao, Y. H., 1976, Scattering Matrix for Elastic Wave, I. Theory, Journal of the Acoustical Society of America, Vol. 60, No. 3, pp. 556-566.

6 Varadan, V. V. and Varadan, V. K., 1979, Scattering Matrix for Elastic Wave, III. Application to Spheroids,'' Journal of the Acoustical Society of America, Vol. 65, No. 4, pp. 896-905.

7 Yeh, Y. K. and Pao, Y. H., "On the transition matrix for acoustic waves scattered by a multilayered inclusion", J. Acoust. Soc. Am., Vol.81, pp.1683-1687 (1987).

8 Chai, J.-F., Teng, T.-J., and Yeh, C.-S., 1998, Determination of Resonance Frequency of Two-Dimensional Alluvial Valley by Background Subtraction Method, The Chinese Journal of Mechanics, Vol. 14, No. 1, pp. 1-15.

9 Lee, V. W. and M. D. Trifunac, "Response of tunnels to incident SH-waves," J. Engng. Mech. Div., ASCE, Vol. 105, pp. 643-659 (1979).

10 El-Akily, N. and S. K. Datta, "Response of circular cylindrical shell to disturbances in a half-space," Earthq. Engng. Struct. Dynam., Vol. 8, pp. 469-477 (1980).

11 Wong, K. C., A. H. Shah and S. K. Datta, "Dynamic stresses and displacements in buried tunnel," J. Engng. Mech. Div., ASCE, Vol. 111, pp. 218-234 (1985).

12 Luco, J. E. and de Barros, F. C. P., "Seismic response of cylindrical shell embedded in a layered viscoelastic half-space I. Formulation, II. Validation and numerical results," Earthq. Engng. Struct. Dynam., Vol. 23, pp. 553-580 (1994).

13 Yeh, C.S., Teng, T.J., Shyu, W.S. and Tsai, I.C., "A hybrid method for analyzing the dynamics responses of cavities of shells buried in an elastic half-plane," The Chinese Journal of Mechanics, Vol. 18, no. 2, pp. 75-87 (2002).

14 Chen, J.T., Chen, C.T., Chen, P.Y., and Chen, I.L., "A semi-analytical approach for radiation and scattering problems with circular boundaries," Computer Methods in Applied Mechanics and Engineering, Vol. 196, pp.2751-2764 (2007).

15 Chen, J.T., Shen, W.C., and Wu, A.C., "Null-field Integral Equations for Stress Field around Circular Holes under Antiplane Shear," Engineering Analysis with Boundary Elements Vol. 30, pp. 205–217 (2006).

16 W. I. Liao, Dynamic Responses of an Elastic-Half Space with Defect Subjected to Elastic Waves. PhD thesis, National Taiwan University, 1997.(in Chinese)

17 Yeh, C. S., Teng, T. J., Liao, W. I., and Tsai, I. C., "A series solution for dynamic response of a cylindrical shell in an elastic half-plane", The 3rd National Conference on Structural Engineering, Kenting, Taiwan, pp. 1553-1562 (1996).

Figure 3 Dynamic responses for (a) horizontal displacements, (b) vertical displacements, and (c) hoop stresses along inner surface of a buried tunnel subjected to incident P-wave with .(solid line and dash line are results of  and , respectively. Circles are the results obtained by Yeh et al. [17])

Figure 4 Dynamic responses for (a) horizontal displacements, (b) vertical displacements, and (c) hoop stresses along inner surface of a buried pipeline subjected to incident SV-wave with .(solid line and dash line are results of  and , respectively. Circles are the results obtained by Yeh et al. [17])

Figure 5  Dynamic responses for (a)-(c) displacement amplitude, and (d)-(f) stress amplitudes along inner surface of a buried pipeline subjected to obliquely incident P-wave with  and . (solid line and dash line are results of  and , respectively.)

Figure 6  Dynamic responses for (a)-(c) displacement amplitude, and (d)-(f) stress amplitudes along inner surface of a buried pipeline subjected to obliquely incident SH-wave with  and . (solid line and dash line are results of  and , respectively.)